FieldStruts

User Manual

Rescue Strut Selection, Inventory Management
& Shoring Operations for USAR/FEMA

Version 3.22

2026-06-07

https://vergo402.github.io/paratech-struts/

# Table of Contents

1.	Getting Started

2.	Quick Find

3.	Operations

4.	Shore Points

5.	Command

6.	Cut Table

7.	Inventory

8.	Settings

9.	Safety & Updates

10.	Offline Use

11.	Version History

# 1. Getting Started

## Installing the App

FieldShore is a Progressive Web App (PWA). No app store required.

1.	Open https://vergo402.github.io/paratech-struts/ on your phone or tablet.

2.	iOS: Tap the Share button → "Add to Home Screen."

3.	Android: Tap the browser menu → "Add to Home Screen" or accept the install prompt.

4.	The app icon appears on your home screen and launches fullscreen.

## Connecting Your Department

On first launch, you'll see the login screen asking for a Department ID.

•	Enter your department's shared ID (e.g., engine42, hfd217). No password needed.

•	This ID connects your device to shared inventory and operations via Firebase.

•	All devices using the same Department ID see the same data in real time.

•	You can change departments later from Settings → Log Out.

If you don't have a Department ID yet, enter any short identifier. Share the same ID with your crew so everyone syncs.

The Department ID login screen.

## Using FieldShore on Desktop

FieldShore is mobile-first, but it adapts to laptops and command-post screens. When the browser window is 1024px wide or wider (most laptops and desktop monitors, plus iPad in landscape), the app switches into desktop view:

•	Top nav bar. The five tabs (Quick Find, Operations, Command, Inventory, Settings) move from the bottom of the screen to a horizontal bar at the top.

•	Wider canvas. The app expands to 1200px wide instead of the phone-width 600px column.

•	Split Command tab. Your roster (Apparatus, External Equipment, Individuals, My Role, Hazards) stays pinned on the left while the Dashboard, ICS Organization (org chart), and Layout fill the right pane. The op title bar spans across the top.

•	Split Operations tab. A drilldown sidebar on the left shows your Buildings → Divisions → Areas as a clickable tree, with live status counts on each node (e.g., a yellow 3 next to a division means 3 shore points are currently at Strut Installed). A search box above the tree filters shore points by label, building, division, or area in real time. The right column shows the filtered shore points, with the + Shore Point button centered above the status groups. Mobile uses the inline breadcrumb instead.

Switching to mobile view on a desktop. A small toggle button sits in the upper-right corner of the viewport. Click it to flip the app back to the familiar phone layout (centered narrow column, bottom nav). The toggle icon swaps between a phone glyph and a monitor glyph depending on the current mode. Your choice is remembered — the next time you load FieldShore on the same browser, it opens in the layout you last chose.

Phones and iPad in portrait orientation always use the mobile layout — the toggle button is hidden because there's nothing to switch to.

Desktop view — Command tab split layout.

Desktop view — Operations tab with drilldown sidebar.

# 2. Quick Find

The Quick Find tab is the fastest way to find which Paratech struts fit a given measurement.

## How to Use

1.	Enter the Opening Measurement in feet, inches, and fraction (down to 1/16").

2.	Optionally enter an Estimated Load in pounds.

3.	Select which strut systems you have available: Gold (LongShore), Grey (AcmeThread), and/or LockStroke. If none are selected, all types are shown.

4.	Tap Find Struts.

Quick Find — entering a measurement and load.

## Results

The app returns all strut and extension combinations that fit the measurement, ranked by safety factor. Each result card shows:

•	Strut model name and system (Gold/Grey/LockStroke)

•	Extension (if needed) and combined length range

•	Load capacity at 4:1 safety factor

•	Whether the combination is within safe working limits for the entered load

Quick Find results, ranked by safety factor.

## Deductions

Toggle Include Deductions to account for header wood, footer wood, and connector plates in the measurement:

•	Header/Footer: None, 4x4 (3.5"), or 6x6 (5.5")

•	Top/Bottom Plates: Select from 14 Paratech connector types, each with a specific height deduction

The app subtracts these from your opening measurement before searching for struts, so the results reflect actual strut length needed.

## Inventory Quick View

Tap the cube icon (bottom-right floating button) anytime to see your current available inventory without leaving the screen.

## Quick Start (Start an Operation Without the Modal)

When no operation is active, a Quick Start button appears in the bottom-right corner of the Quick Find tab.

How it works:

•	Press and hold for 500ms. A circular progress arc fills around the button as you hold. Release before it completes and nothing happens — no accidental starts.

•	When the arc completes, the app immediately creates a new operation. The operation name is set to today's date and time (e.g., 05/19/26 @ 14:32). You are set as IC. The Start Operation modal is skipped entirely.

•	If you had a measurement entered in Quick Find, the Add Shore Point form opens with that measurement and load already filled in. If Quick Find was empty, it opens a blank Add Shore Point form.

•	You can rename the operation later from the Command tab.

•	On a keyboard, press Enter or Space to fire immediately without holding.

The button is hidden while any operation is active, and while any other modal is open.

# 3. Operations

The Operations tab is focused on the shore-point workflow — adding shore points and moving them through the status sequence. Apparatus, personnel, and org chart are on the Command tab (see Section 5).

## Starting an Operation

1.	Tap Start New Operation.

2.	Enter the Operation Name (e.g., "Building Collapse — 123 Main St").

3.	Optionally enter a Task Force name.

4.	Check Multi-Building if the operation spans more than one structure.

5.	Select which apparatus to assign (all are checked by default).

6.	Tap Start Operation. The app drops you directly onto the Operations tab, ready to add shore points.

The Start New Operation form.

The Operations tab with an active operation.

## Ending an Operation

The End Operation button is on the Command tab, in the header next to the operation name. It is visible only to the device assigned as IC.

Tap End. A confirmation sheet opens. Type END (all caps) and tap the now-unlocked End Operation button. This two-step gate prevents accidental end-of-op from a stray tap — especially with gloves on.

The operation moves to the Archived Operations list at the bottom of the Operations tab, where you can view or delete it. The app automatically backs up the operation before ending it.

# 4. Shore Points

Shore points are the core of an operation — each one represents a location that needs shoring.

## Adding a Shore Point

1.	Tap + Shore Point.

2.	Enter a Label (e.g., "SP-1", "Kitchen Wall").

3.	If multi-building is enabled, enter a Building name.

4.	Pick the Division from the numbered dropdown — see Numbered Divisions below. Enter the Area (specific location).

5.	Select the Group (which apparatus is responsible).

6.	Choose a Shore Type:

•	Vertical T-Shore — Single strut with header and footer

•	Double-T Vertical Shore — Two struts with shared header

•	3-Post Vertical Shore — Three struts with 6x6 header and footer

7.	Enter the Opening Measurement and optional Load.

8.	Set Quantity (1-4 struts).

9.	Tap Find & Deploy to search inventory and deploy matching equipment.

The Add Shore Point form.

## Shore Point Lifecycle

Each shore point progresses through six statuses:

| Status | Meaning |
| --- | --- |
| Pending | No equipment available — saved for later |
| In Process | Equipment deployed, work starting |
| Strut Installed | Strut is in position |
| Cutting | At the cut table for final sizing |
| Runner | Being transported to location |
| Secured | In place and operational |
| Removed & Returned | Shore deconstructed, equipment returned to inventory |

Use the status buttons on each shore point card to advance or send back through the workflow. Role gates apply to several actions:

•	Send to Runner — Runner role only (plus IC override). Other roles see the button disabled with a tooltip.

•	Mark Secured — IC, Safety Officer, and Shoring roles (Initial Shoring, Wood Shoring, Shoring Lead) only. Other roles see the button disabled with a tooltip.

•	Mark Cut Complete — Cutting role only (plus IC override).

•	Remove & Return Equipment — Entry, Rescue, or Initial Shoring roles (plus IC override).

If your role does not match, the button shows greyed with a tooltip explaining what is required. Switch your role in My Role (Command tab) or have a teammate with the right role advance it. IC overrides all gates.

When the strut finder offers a configuration in the LongShore unrated zone (>16 ft), the deploy button shows a final confirmation modal — Cancel or "I Acknowledge — Deploy" — before the shore point is created. The acknowledgement, including who tapped it and when, is recorded on the shore point for after-action review.

A shore point card with its status badge and actions.

## Pending Shore Points

When no matching equipment is available, shore points are saved as Pending with a purple status badge. The app monitors inventory and shows a green "Equipment now available!" indicator when a match appears (e.g., after equipment is returned or external equipment is added).

Tap Assign Equipment on a pending card to open the strut picker pre-filled with that shore point's measurement, load, and deductions. The finder runs automatically against the current operation inventory; pick a strut to deploy. The pending card transitions to In Process and inventory decrements — the shore point keeps its label and location.

## Grouped Shore Points

Shore types with quantity > 1 (e.g., a 3-strut T-shore) create grouped shore points. Early status transitions (In Process → Strut Installed → Cutting) advance all members together, since the physical shore system is set up as a unit. Once in the cutting workflow, each card tracks independently — wood is cut, sent to the runner, secured, and returned one piece at a time.

## Drill-Down Navigation

Shore points are organized by hierarchy: Building > Division > Area > Group. Tap into each level to filter, with breadcrumb navigation and a back button at each step.

## Numbered Divisions

As of v3.15, division is a numbered dropdown instead of a free-text field. This keeps your shore-point labels consistent across the whole crew and removes radio ambiguity ("Bsmt" vs "Basement" vs "B-Level" → all the same).

Reading the picker. The dropdown reads top-to-bottom matching the building cross-section:

•	Div 3 (+2 floors up), Div 2 (+1 floor up) — higher floors at the top

•	Div 1 (Ground level) — anchor row, always present

•	Sub Div 1 (Basement), Sub Div 2 (+1 below) — basement and sub-basements at the bottom

Adding floors. Use the two buttons below the dropdown:

•	+ Floor Above — adds the next Division (Div 2, then Div 3, etc.)

•	+ Sub Div Below — adds the next Sub Division (Sub Div 1 = Basement, then Sub Div 2, etc.)

There is no manual text entry. The list is shared across the operation — every shore point on the same op picks from the same set.

Upgrading from older operations. If your operation was created on v3.14 or earlier, the app automatically converts existing division labels:

•	Numeric labels ('1', '2') become Div 1, Div 2, etc.

•	'Ground', 'G' becomes Div 1.

•	'Basement' becomes Sub Div 1.

•	Anything else ('A', 'Alpha', 'Mezzanine B') is preserved as a legacy label on each shore point.

When legacy labels are present, an orange banner appears above the shore-point list with a Show legacy SPs button. Open each of those shore points and re-pick a numbered division — saving clears the legacy label.

The renumber prompt only appears when the operation has been quiet for 30+ minutes; it never interrupts an active incident.

# 5. Command

The Command tab is the Incident Commander's dedicated screen. It holds everything needed to manage the command structure — apparatus, personnel, the org chart, dashboard stats, and the hazard log.

A runner working shore-point status works in the Operations tab. The IC reviewing staffing and hazards works here. Each role gets a focused surface.

## What's on the Command Tab

•	Dashboard tiles — Three at-a-glance tiles at the top: Elapsed (HH:MM:SS timer, counts up from operation start), Apparatus (count), and Shore Points (total). The timer ticks every second. Three-tile row on desktop; stacks vertically on phone.

•	End Operation — In the header, next to the operation name. Visible to IC only. Tap to end the operation (see Ending an Operation).

•	Assigned Apparatus — Which apparatus are part of this operation, grouped by category (Chief, Deputy Chief, Engine, Ladder, Rescue, Squad, etc.). Each category renders as a labeled block with its chips beneath. Tap Assign to add/remove, or Group to organize apparatus into named task-force groups.

•	External Equipment — Equipment borrowed from other departments or agencies. Tap + Add to log borrowed gear (see below).

•	Individuals — Named personnel assigned to the operation. Tap + Add to enter a name.

•	My Role — Set your device's ICS role. This determines which view your device defaults to.

•	ICS Organization — The full org chart (see below).

•	Hazard Log — Log and track scene hazards (see below).

The Command tab — dashboard tiles and sections.

## Adding External Equipment

Tap + Add in the External Equipment section to log gear borrowed from another department or agency.

The form matches the regular inventory-add layout. At the top, enter the Source Department and Source Apparatus. Then use the quick-add grid to select struts, extensions, and connector plates — the same grid used in regular inventory. Tap any item to add it; the quantity badge increments as you tap. Tap Done when finished.

External equipment added here tracks available vs. deployed quantities the same way regular inventory does. If a return fails while offline or when the connection drops, the change is queued and reconciled automatically when connectivity is restored.

Where external equipment shows up (v3.20):

•	Quick View "Available Inventory" sidebar — external items appear in a dedicated External Department Equipment section at the bottom of the sidebar. Below each item label, the source department and apparatus are shown on a 11px subordinate sub-label (e.g., "Yonkers FD / Engine 5"). Deployed items disappear from this view once their available count hits zero.

•	Deploy flow — external struts, extensions, and connector plates are all selectable when you deploy a shore point (v3.20 closed a bug where external extensions weren't findable in the deploy modal).

•	Inventory tab — external equipment does NOT appear here. The Inventory tab is for managing your department's owned gear (apparatus inventory, Excel import/export). Visiting-dept gear lives on Command + Quick View only.

•	Demob accountability — when a visiting dept demobs, deployed external items live on their shore-point cards and in the Command tab External Equipment list, not in Quick View (Quick View hides items with zero available count, which is correct for "what can I pull right now" but means it isn't a demob tally surface). Cross-reference the Command list against deployed shore points to confirm what to hand back.

## ICS Organization Chart

The org chart renders as an interactive tree with L-shaped connector lines showing the command hierarchy at a glance.

### Default Roles

The default ICS hierarchy is:

Incident Commander (IC)
├── Safety Officer
└── Operations
    ├── Staging Area Manager
    ├── Division 1                          (collapsed by default)
    │   ├── Entry
    │   ├── Rescue
    │   ├── Initial Shoring
    │   └── Wood Shoring
    └── Cutting Table
        └── Runner

Division 1 is collapsed by default to fit the chart on a phone screen. Tap the chevron (▸) on the Division 1 card to expand the four tactical roles when you're ready to assign them.

Staging Area Manager reports to the Operations Section Chief per NIMS doctrine — not as Command Staff. Use the org chart's rearrange controls (unlock with the 🔒 Edit button) if your incident structure requires Staging visible at IC level.

Card titles can be up to 30 characters and word-wrap centered in the card. The smaller subtitle line was removed in v3.18.2 — the title now stands alone.

The ICS Organization chart (default hierarchy).

### Assigning Roles

•	Tap any node to open the role management modal.

•	Assign available apparatus or individuals to a role.

•	Each role shows who's assigned and their status.

•	When you assign yourself to a role, a confirmation toast appears (e.g., "You are now Incident Commander").

### Status Indicators

Filled roles display a status dot:

•	Green (circle) — Active: role has assignees AND at least one shore point is in progress under this role's area.

•	Amber (square) — Staged: role has assignees but no active shore points (crew assigned, waiting).

•	A legend below the org chart header explains the dot meanings.

### Headcount Badge

The header shows a resource count (e.g., "3/5 resources assigned") so you can see staffing at a glance.

### Span of Control

Two warning tiers, per NIMS guidelines (optimal span of control is 3–7):

•	Yellow ⚠ — Approaching limit: 6–7 direct reports. Keep an eye on it.

•	Red ⚠ — Exceeded: more than 7 direct reports. Split this branch.

### Custom Roles

•	Tap any node → + Sub-Role to add a child role underneath it.

•	Tap any node → Rename to change its display name.

•	Custom roles can be removed (along with any sub-roles) via the node modal.

•	The + Role button in the org chart header adds a new role under IC.

### Reordering and Reparenting

Each card has an inline toolbar with controls to rearrange the hierarchy:

•	↑ Up / ↓ Down — Move a role earlier or later among its siblings.

•	↰ Promote — Move a role up one level in the hierarchy (closer to IC).

•	↳ Demote — Nest a role under its previous sibling.

On phones (under 480px wide), the inline toolbar is hidden to save space. The same controls appear in the modal when you tap a node.

Other reparenting options:

•	Long-press drag (mobile): Hold a role for 500ms. An orange highlight and haptic feedback confirm reparent mode. Drag to the new parent and drop.

•	"Move to..." menu (all devices): Tap a role → Move to... → select the new parent from the list → confirm.

After reparenting, a toast notification appears with an Undo link.

Locked roles: Incident Commander and Safety Officer cannot be moved. IC is always the root; Safety always reports directly to IC (NIMS requirement).

Access control: Only the device assigned as IC can add, remove, or rearrange roles.

### Swapping Roles

A short drag (not a long-press) between two roles swaps their personnel assignments, not the hierarchy. This is useful for crew rotations.

## My Role

Set your device's ICS role from the My Role section. Your role controls your default view on launch:

•	IC / Safety → Command tab

•	Operations roles → Operations tab

•	Cutting Table → Cut Table view

## Hazard Log

The Hazard Log section lets you document scene hazards during the operation. It supports your Incident Action Plan and post-incident review. It is not a substitute for radio communication — critical hazards still need to go over the air to all crews.

### Logging a Hazard

1.	Tap + Add Hazard in the Hazard Log section.

2.	Select a Type: Structural Instability, Utility, Atmospheric, Fall, or Other.

3.	Enter a Location — describe it in plain words (e.g., "North wall, second floor").

4.	Set Severity: Low, Medium, or High.

5.	Add any Notes (optional).

6.	Tap Save. The hazard is recorded with your name and the time.

Open hazards appear at the top of the list, sorted high-to-low severity.

### Mitigating a Hazard

Tap Mitigate on a hazard card when the condition is controlled. The log records who mitigated it and when. Tap Reopen if the hazard returns.

Hazards stay with the operation record. They appear in the archived op after end-of-op.

# 6. Cut Table

The Cut Table sub-tab is a focused view for the cutting station. It shows all shore points currently in the Cutting status with their required measurements, making it easy to see what needs to be cut without scrolling through the full operations list.

Each shore point card has its own Mark Cut Complete button — even within a group, each piece of lumber is tracked individually. Once marked, the card moves to the Cut Complete section where it can be sent to the runner independently.

The Cut Table view.

# 7. Inventory

The Inventory tab manages your department's strut, extension, and connector plate inventory across apparatus.

The Inventory tab — apparatus and Quick Add grid.

## Adding Apparatus

1.	Go to Inventory → tap + Apparatus (or go to Settings).

2.	Enter a name (e.g., "Engine 42") and select a type (Chief, Engine, Ladder, Rescue, Squad, Task Force, Other).

3.	Tap Add.

## Adding Equipment

For each apparatus:

1.	Tap the apparatus name to expand it.

2.	Tap + Add to open the equipment modal.

3.	Use Quick Add for fast entry — a visual grid of all Paratech strut models, extensions, and connector plates. Quantity badges show how many you've already added.

4.	Or use Manual Add to enter a custom item.

## Editing Quantities

•	Use the + and - buttons on each inventory item to adjust quantities.

•	Items with deployed equipment cannot be reduced below the deployed count.

## Excel Import/Export

•	Export: Tap the export button to download your full inventory as an Excel (.xlsx) file.

•	Import: Tap the import button to upload an Excel file. The app reads the columns and merges with existing inventory. An ID column preserves item references across round-trips.

## Custom Apparatus Types

Departments can customize the apparatus type list from Settings → Apparatus Types:

•	Add new types (e.g., Hazmat, Brush, Medic)

•	Rename existing types

•	Reorder with arrow buttons

•	Remove unused types (blocked if apparatus still use the type)

# 8. Settings

The Settings tab includes:

•	Department Name — Display name for your department.

•	Department ID — Your current connection ID. Tap Log Out to disconnect and enter a different ID.

•	Appearance — Choose System (follows your device), Light, or Dark theme.

•	Apparatus Types — Customize the apparatus type list for your department.

•	Check for Updates — Manually tells the app to look for a new version right now. Useful if dispatch announces a hotfix and you don't want to wait. The app normally checks every 5 minutes in the background, so this is rarely needed.

•	Feedback — Submit bug reports or feature requests.

•	Data Management — Options for managing local and synced data.

The Settings tab.

# 9. Safety & Updates

## Measurements Round Down to 1/8"

Cut lengths and strut set-to lengths now round down to the nearest 1/8 inch — you will never be told to cut or set *long*. When a length lands between marks it is shortened, because the loading wedge takes up that small gap, while a piece cut too long cannot seat. The deduction math still works from the exact plate and wood sizes; only the final number you act on is rounded, once, to a tape-readable 1/8".

## "Verify Physical Strut" Flag

If two crews deploy against the same strut at the same time while offline, the app could briefly count one strut as two. When the devices reconnect and the app detects this, it flags the affected shore point with a persistent amber VERIFY PHYSICAL STRUT badge instead of silently hiding the discrepancy. The flag stays until the IC or Safety taps Acknowledge — go put eyes on that shore point and confirm a real strut is in place before relying on it. A flagged shore point is not counted as secured on the Command dashboard until it is acknowledged.

## Ending an Operation — Confirmation Gate

The End Operation button lives on the Command tab header and is visible to the IC only.

Ending an operation requires you to type END before the button unlocks. This prevents an accidental end-of-op from a stray tap when you're reaching for something else on the screen — a real risk with gloves on.

The app still creates an automatic backup of the operation before ending it. The type-gate is an extra layer at the tap, not the recovery net.

## Force Update

If a critical fix ships, a non-dismissible "Update Required" screen appears when you launch the app. There is one button: Reload Now.

•	Tap it. The update takes about 3 seconds.

•	Your local data — inventory, settings, any active operation — is preserved across the reload.

•	You cannot use the app until the update is applied.

This screen only appears when an administrator has flagged a version as no longer safe to run. It is not a routine update prompt.

## Routine Updates

For normal updates, see Section 10 (Offline Use / Updating the App).

# 10. Offline Use

FieldShore is designed for field use where connectivity is unreliable.

•	The app caches all assets locally and works fully offline after the first load.

•	All data changes save to your device first, then sync to your team when a connection is available.

•	If you lose connection mid-operation, keep working normally. Changes queue up and sync automatically when connectivity returns.

•	The offline banner shows exactly how many changes are waiting to sync (e.g., "Offline — 5 changes queued, will sync when reconnected"). This tells you your data is safe even without a connection.

## Updating the App

When a new version is available, the app downloads it in the background. To activate the update:

•	Close and reopen the app, or

•	Refresh twice — the first refresh downloads the new version, the second activates it.

To check immediately, go to Settings → Check for Updates.

# 11. Version History

Major and minor releases only. Patch releases (bug fixes) are omitted.

| Version | Date | Highlights |
| --- | --- | --- |
| v3.22 | 2026-06-07 | Inventory-integrity & measurement-safety release. Measurements now round _down_ to 1/8" (cut and set-to lengths) so you are never told to cut long — short is taken up by the wedge (issue #300). Over-allocation safety flag: if two crews deploy the same strut at once while offline, on reconnect the app flags the shore point with a persistent VERIFY PHYSICAL STRUT badge (cleared only by IC / Safety) instead of silently losing count, and a flagged point is not counted as secured (issue #80). Under the hood: stronger validation on visiting-department equipment counts, and a clean-up of the inventory deploy/return code with no change in behavior (issue #284). _(Header/footer beams and the Command department breadcrumb were planned for this release but deferred.)_ |
| v3.21 | 2026-05-25 | All measurements now display as fractions (issue #119, Hartsdale field feedback). Shore-card lengths, deduction summaries, cut table expected/actual, archived ops, Quick Find results, and Quick View inventory all render in tape-measure format — 48-1/2" instead of 48.5" — at 1/16" precision. The "Actual cut" override on the cut table is now a feet/inches/fraction picker matching the rest of the app, so cutters don't have to mentally convert tape-measure readings into decimals. Partially-typed actual measurements survive Firebase status pushes from peer devices. Excel import now accepts fractional text like 48 1/2 in the Extension Length column (previously silently truncated to 48). Excel exports remain in decimal for spreadsheet math compatibility. |
| v3.20 | 2026-05-25 | External equipment now flows into "available inventory" end-to-end (issue #127). Adding external struts, extensions, or plates on the Command tab now makes them deployable on shore points — previously external extensions were silently invisible to the deploy modal, and external plates had a pre-existing silent loss on return. External items also now appear in the Quick View "Available Inventory" sidebar in a dedicated External Department Equipment section, with the source dept and apparatus shown on a subordinate sub-label under each item. The Inventory tab is unchanged — it remains dept-owned-only by design (visiting-dept gear is op-scoped and lives on Command + Quick View). |
| v3.19 | 2026-05-20 | Default ICS org chart restructured for NIMS Type IV–V incidents. Added two new default roles: Staging Area Manager (under Operations per NIMS doctrine) and Division 1 (under Operations, collapsed by default to fit phone screens). The four tactical roles — Entry, Rescue, Initial Shoring, Wood Shoring — now sit under Division 1 instead of directly under Operations. Cutting Table and Runner stay where they were. Card titles (v3.18.2 fix carried forward) accept up to 30 characters with word-wrap and no subtitle row. |
| v3.18 | 2026-05-20 | hfd217 field-feedback response. Removed: scenario presets in Start Operation (added cognitive load at incident-start) and the Solo-IC / Auto-promote mode block on Command (exposed implementation state as user concept). The Command tab now always renders the full view regardless of apparatus count. Added: three dashboard tiles at the top of Command — Elapsed (HH:MM:SS, ticks every second), Apparatus count, Shore Points count. Fixed: cutting table Send to Runner action restored (workflow blocker). Operations Section Chief role can now mark cut-done and send to runner (battalion-chief doctrine — Ops keeps physical-custody actions off the Command tab). Adding external equipment now updates the available inventory display immediately on both the Command and Inventory tabs. Operations tab header no longer shifts left when the inventory right-rail is on. Assigned Apparatus column now groups chips under per-category labels (Chief, Deputy Chief, Engine, Ladder, etc.) instead of running labels inline. Plate picker sorts in-stock plates to the top with "Available" / "Not in inventory" section labels. |
| v3.17 | 2026-05-19 | "Strut Placed" renamed to "Strut Installed" (FEMA US&R FOG) everywhere in the UI — badges, buttons, legend, drilldown counts. Quick Start FAB on the Quick Find tab: press and hold 500ms to instantly create an operation and jump to the first shore point, skipping the Start Operation modal entirely. End Operation moved to the Command tab header — IC only. Send to Runner now requires the Runner role (plus IC override). Mark Secured now requires IC, Safety, or a Shoring role (Initial Shoring, Wood Shoring). Add External Equipment form rebuilt to match the regular inventory-add layout — struts, extensions, and connector plates in one quick-add grid with Source Department and Source Apparatus fields at top. External equipment returns now reconcile automatically after offline use or connection failures. _(Scenario presets and Solo-IC mode shipped in v3.17 but were removed in v3.18 after field feedback — see v3.18 row.)_ |
| v3.16 | 2026-05-19 | SmartArt-style ICS org chart. The command hierarchy now renders as a visual tree with L-shaped connector lines between parent and child roles. Each card has an inline toolbar with reorder controls (Up, Down, Promote, Demote) so you can restructure the hierarchy without drag-and-drop. On phones the toolbar is hidden and the same controls appear in the node-tap modal. Only the IC can add, remove, or rearrange roles (changed from IC+Safety per NIMS SM-0322). Role assignment now shows a confirmation toast. All org chart buttons meet 44px minimum touch targets for gloved use. |
| v3.15 | 2026-05-19 | Numbered divisions with vertical anchoring. Division is now a top-to-bottom picker — Div 3 / Div 2 / Div 1 (Ground) / Sub Div 1 (Basement) — with + Floor Above and + Sub Div Below buttons. No manual text entry; the division list is shared across the operation so labels stay consistent. Existing operations migrate automatically (numeric and 'Ground'/'Basement' labels convert in place; anything else is preserved as a legacy label with a non-blocking review banner). Offline inventory hardening: deploys and returns made while offline now reconcile correctly with Firebase on reconnect via a per-item touched-set flush-pass (closes the v3.8.2 architectural root cause). Concurrent-deploy race guard — when another device takes the last unit, the second device sees "Another user took the last X — try again" and the orphaned shore-point write is rejected. |
| v3.14 | 2026-05-18 | Operations tab on desktop now splits into two columns. Left sidebar (sticky) shows a drilldown tree of Buildings → Divisions → Areas with live status counts on each node, and a search input above the tree filters by label / building / division / area. Right column shows the filtered shore points with the + Shore Point button centered above the status groups. Mobile is unchanged (inline breadcrumb stays). |
| v3.13 | 2026-05-18 | Desktop view. On screens 1024px wide and up the app now uses a horizontal top nav bar and widens the container to 1200px. The Command tab splits into two columns on desktop — roster (Apparatus, External, Individuals, My Role, Hazards) on the left (sticky), Dashboard / ICS Organization / Layout on the right. A small toggle in the upper-right of the viewport flips between desktop view and mobile view; the choice is remembered. Phones and iPad portrait are unchanged. |
| v3.12 | 2026-05-18 | Command tab promoted to top-level navigation (apparatus, individuals, org chart, hazard log separated from shore-point workflow). Hazard Log (ICS-208 style) with severity tiers, mitigate/reopen, and archived op persistence. End Operation now requires typing "END" to confirm (glove-safe gate). Force-update overlay for critical fixes (data preserved). Yellow span-of-control warning tier at 6–7 direct reports. Offline banner shows queued write count. Settings → Check for Updates button. |
| v3.11 | 2026-05-17 | Field-feedback round-up. Quick Find plate picker selection fixed (tap on a connector now actually selects it). Header scrolls away with content instead of locking real estate at the top. Edit Apparatus modal no longer shows the Add form simultaneously. Quick-add inventory list preserves scroll position when adding items near the bottom. Pending shore-point length edits now keep effective length in sync when deductions are toggled off, with success toast. Comprehensive dark-mode contrast pass — buttons, plate pickers, apparatus tabs, qty controls, and small action buttons are all theme-aware (no more black-on-dark text). |
| v3.10 | 2026-05-16 | LongShore unrated-zone deploys now require an explicit acknowledgement modal (audit trail captures who/when). Pending shore points have a real "Assign Equipment" deploy flow. Locked-card status buttons are role-gated (Cutting / Runner / Entry-Rescue-Shoring, with IC / Safety override). Excel imports warn before orphaning deployed equipment references. Persistent banner on auth-failure / sync degradation with manual retry. SheetJS bundled into the service worker for offline import/export. Fully-extended boundary warning on strut results. |
| v3.9 | 2026-05-16 | Group status guard — sending one shore point back through the workflow no longer drags advanced group-mates with it. Excel imports support extensions and connector plates (not just struts). Firebase scripts pinned with Subresource Integrity. Stored XSS hardening on shore point card rendering. |
| v3.8 | 2026-05-16 | Individual wood cut tracking per shore point card (grouped points no longer share cut/runner/secured lifecycle). Inventory tab updates immediately after deploying or returning equipment. Sync diagnostics for offline write queue troubleshooting. |
| v3.7 | 2026-05-15 | Firebase Anonymous Auth + security rules. Photo attachment in feedback form. Status dot key/legend on ICS org chart. Liability disclaimer (removed). Clearer empty state when inventory lacks a fitting strut. Conservative-floor interpolation for load capacity. |
| v3.6 | 2026-05-15 | Firebase listener cleanup (no more leak on department switch). Concurrent edit safety for org chart swaps. Keyboard accessibility for all interactive elements. Performance optimization for apparatus name lookups. |
| v3.5 | 2026-05-14 | Dark/light/system theme toggle. Shore point Group field changed to apparatus dropdown. Local-first write architecture across all mutation sites. |
| v3.4 | 2026-05-14 | Command page overhaul: role reparenting (long-press drag + "Move to..." menu), collapsible branches, status indicators (active/staged), headcount badge, span-of-control warnings. |
| v3.3 | 2026-05-14 | 6-pass production-readiness audit. App renamed from "Paratech Strut Selector" to FieldStruts. Extension rule fixes, error handling improvements, safety checks in selection algorithm. |
| v3.2 | 2026-05-11 | Customizable apparatus types (add, rename, reorder, remove) from Settings. |
| v3.1 | 2026-05-11 | Dynamic ICS org chart: custom sub-roles, rename roles, remove roles, tree-based rendering at any depth. |
| v3.0 | 2026-05-11 | Drag-and-drop ICS org chart repositioning (tap-to-swap, desktop drag, touch drag). |
| v2.4 | 2026-05-11 | Apparatus type hierarchy (Chief through Other). Apparatus groups for combining units during operations. |
| v2.3 | 2026-05-11 | NIMS/ICS terminology update ("Team" to "Group"). Collapsible operation sections. Quick Find defaults to no system preselected. |
| v2.2 | 2026-05-11 | Interactive ICS org chart (tap any node to assign). Grouped shore points advance together. |
| v2.1 | 2026-05-11 | Cut measurement persistence. Actual vs. expected cut display. Label clarity improvements. |
| v2.0 | 2026-05-11 | Production hardening: Firebase error handling with offline write queue, XSS protection, safe localStorage writes. |
| v1.11 | 2026-05-10 | Pending shore points (save when no equipment available, auto-detect availability). ICS/NIMS hierarchy chart. |
| v1.10 | 2026-05-10 | Individual personnel tracking with ICS role assignment. |
| v1.9 | 2026-05-10 | External equipment editing after creation. Quick Add grid for external equipment. Drill-down stops at Area level. |
| v1.8 | 2026-05-10 | 6-status shore point workflow (In Process through Removed & Returned). |
| v1.7 | 2026-05-10 | ICS drill-down navigation (Building > Division > Floor > Team). 3-tier view system (Operations, Command, Cut Table). |
| v1.4 | 2026-05-08 | Live inventory count badges on Quick Add. |
| v1.1 | 2026-05-07 | Department login screen. Multi-strut deploy (1x-4x per shore point). |
| v1.0 | 2026-05-07 | First release. Quick Select, apparatus management, inventory tracking, live operations. |