FieldShore

User Manual

Rescue Strut Selection, Inventory Management
& Shoring Operations for USAR/FEMA

Version 4.0

2026-10-17

Your Admin will give you the web address when FieldShore 4.0 goes live.

# About This Manual

FieldShore helps a rescue crew do three jobs on a phone. Pick the right Paratech strut for an opening. Track every shore point from first measurement to gear back on the rig. Keep count of the gear on each apparatus.

FieldShore keeps working with no signal. It saves to your phone first. It saves to the cloud when it can.

This manual covers FieldShore 4.0. Version 4.0 replaces the earlier FieldShore (version 3). Many screens and steps changed. If an old habit does not match this manual, trust this manual.

FieldShore is a planning aid. It is not an engineering certification. Check every strut length, load and shore with your team and with rescue engineering.

## Words Used in This Manual

| Word | What it means |
| --- | --- |
| Operation | One incident. You can have one active operation at a time. |
| Shore point | One spot you are shoring. It has a number, like #4, that your crew can call on the radio. |
| Division | A floor of the building. Division 1 is ground level. Sub Division 1 is the basement. |
| Apparatus or rig | A truck or unit that carries gear. |
| Department | Your crew's shared space in FieldShore. Everyone in it sees the same operations and inventory. |
| Admin | A department member who can manage people, roles and settings. |
| IC | Incident Commander. The person in charge of the operation. |
| OP | Operational period. OP 1 is the first period of an operation. |

## How to Read the Steps

Words in bold are buttons or labels you see on the screen. Steps are numbered. Do them in order.

# Contents

•	Getting Started

•	Quick Find

•	Operations

•	Command

•	Inventory

•	Settings

•	Working Offline

•	Safety Notes

•	What Is Not in 4.0

•	Version History

# 1. Getting Started

## The Welcome Page

The welcome page is a short scroll-through of what FieldShore does. It follows one shore point from a 54-inch opening to a piece of wood cut to length.

•	Your first visit in a web browser opens the welcome page. Tap Open FieldShore to go into the app.

•	Every visit after that opens the app on Quick Find.

•	A direct link, such as the sign-in link from your email, goes straight to the app.

•	The app on your home screen never shows the welcome page.

•	To see it again, tap Settings, then About, then See what FieldShore does. The sign-in screen has the same link.

•	With no signal the page still opens. The video does not play.

The welcome page: a short scroll-through of what FieldShore does.

## Installing FieldShore on Your Home Screen

FieldShore installs from your web browser. You do not need an app store.

1.	Open FieldShore in your phone's browser.

2.	On an iPhone in Safari, tap the Share button, then tap Add to Home Screen.

3.	On an Android phone in Chrome, tap the browser menu, then tap Install app or Add to Home Screen.

4.	Open FieldShore from its new icon. It opens full screen.

Open the app once while you have signal. That lets it save what it needs to work with no signal. After that, it updates itself in the background.

## Creating Your Account and Signing In

You do not need an account to use Quick Find. You need an account for Operations, Inventory, Command and Settings.

To create an account:

1.	Tap Sign in.

2.	Tap Create Account.

3.	Type your Display name. This name goes on every action you take.

4.	Type your Email and a Password.

5.	Tap Create Account.

The first time you reach the app after creating an account, a short intro appears. Tap Next through it, then tap Get started. A Getting started list follows. Tap Done to close it. Skip closes both the intro and the list. You can replay them from Settings, then Help & reference.

On the Getting started list, the rows for Operations, Inventory and Command say Coming soon. Ignore that label. Those tabs already work.

To sign in later:

•	Tap Sign In, type your email and password, then tap Sign In.

•	Or tap Email me a sign-in link. Open the link from the email on the same phone.

•	If you forgot your password, type your email and tap Forgot password?. FieldShore emails you a reset link.

•	Continue as guest takes you back into the app. A guest can only use Quick Find.

To log out, tap Settings, then Account, then Log out. Your work on the phone stays put.

The sign-in screen on the Create Account tab.

## Creating a Department

A department is your crew's shared space. The person who creates it is the first Admin.

1.	Sign in with an account that has no department. The Create department screen opens.

2.	Type your Department name.

3.	Tap Create department.

4.	A panel says your department is ready. It shows a QR code and an invite code. Tap Copy to copy the code.

5.	Tap Done. FieldShore opens on Operations.

Show the QR code or read out the invite code to your crew. They use it to join. You can find the code again under Settings, then Department.

If you create a department with no signal, it is saved on your phone. A banner tells you. The invite code works only after the department reaches the cloud.

Skip for now takes you back without making a department.

The invite code and QR code after you create a department.

## Joining a Department

Ask your Admin for the invite code or the QR code.

1.	Sign in or create an account.

2.	Tap Join an existing department.

3.	Tap Scan QR code and point the camera at the code. Or type the code into Enter it by hand. You can also tap Paste.

4.	Tap Join department. The button turns on when the code is long enough.

5.	A panel says Joined with your department name and your role. Tap Continue.

New members get the Default role. You can read everything and run field work. Your Admin can change your role at any time.

With no signal, a panel says You're offline. Your code is saved on the phone. FieldShore finishes the join when signal returns. A banner says Will join "name" when you reconnect. You can keep working.

If the code does not work, a message shows under the code box. Ask your Admin to check it.

The Join department screen with an invite code typed in.

## If Your Admin Added You

An Admin can create your login for you. They read you your email and a starter password in person.

1.	Open FieldShore and tap Sign in.

2.	Type your email and the starter password. Tap Sign In.

3.	On Set your password, type the starter password again.

4.	Type your new password twice. Use at least 6 characters.

5.	Tap Save and continue.

You cannot skip this screen. The only other button is Sign out.

## Moving Around the App

FieldShore has five tabs.

| Tab | What it is for |
| --- | --- |
| Quick Find | Find the right strut for an opening. Open to everyone. |
| Operations | Run the incident. Add shore points and move them along. |
| Inventory | Count the gear on each apparatus. |
| Command | See who is in charge. Org chart and hazard log. |
| Settings | Your account, your department, themes and help. |

On a phone, the tabs sit in a bar at the bottom. Tap the small arrow on the top edge of the bar to hide it. Tap it again to bring the bar back.

A guest sees only Quick Find and a Sign in button.

## Phone and Desktop Layouts

•	On a phone or a tablet held upright, the tabs sit at the bottom.

•	On a screen 1024 pixels wide or more, a rail on the left holds the tabs. This covers laptops, command-post screens and tablets held sideways. Tap Collapse at the bottom of the rail to shrink it to icons. FieldShore remembers your choice.

•	On a laptop, Command shows the incident summary on the left. The Org Chart or Hazard Log fills the right side.

•	Slide controls turn into single buttons when you use a mouse. See Moving a Shore Point Through Its Steps.

Desktop view: the rail on the left, Command situation and Org Chart side by side.

# 2. Quick Find

Quick Find tells you which Paratech struts fit an opening. Anyone can use it, with or without an account, and with no signal.

Quick Find checks the whole Paratech catalog. It does not look at what is on your rigs. It knows 14 struts in three systems: LongShore, Acme thread and LockStroke.

## Finding a Strut

1.	Tap Quick Find.

2.	Under Measurement, type the whole opening. Use the ft box and the in box. You can put everything in inches. For 6 feet, type 72 in the in box.

3.	Tap the eighth of an inch at or just below your tape reading: 0, 1/8, 1/4, 3/8, 1/2, 5/8, 3/4 or 7/8. If the tape falls between two eighths, take the shorter one.

4.	Optional: tap Deductions to add the wood and plates. See the next section.

5.	Optional: type an Estimated load (lbs). This is the load this opening must carry. FieldShore compares it with one strut and shows how many struts it needs. There is no cap at 4.

6.	Tap Find Struts.

Type the raw opening. Do not subtract anything yourself. The largest opening is 30 feet (360 inches).

The load does not change which struts fit. It flags capacity: each card shows how many struts the load needs. With a load entered, fits that need fewer struts come first. Leave the load blank if you do not know it. With no load entered, FieldShore cannot warn you about capacity. A load can be 0 to 500,000 lbs.

Find Struts stays off until you enter a measurement. The button tells you why.

Quick Find with an opening typed in.

## Adding Wood and Plate Deductions

Deductions are the wood and plates that sit between the strut and the structure. FieldShore takes them off your opening and shows the Required strut length.

1.	Tap Deductions to open it.

2.	Pick Header wood: None, 4×4 or 6×6.

3.	Tap Top plate and pick a connector.

4.	Tap Bottom plate and pick a connector.

5.	Pick Footer wood.

6.	Check Required strut length at the bottom.

A 4×4 counts as 3½ inches. A 6×6 counts as 5½ inches. Each connector has its own height.

The result rounds down to the nearest eighth of an inch. It never rounds up.

If the deductions use up the whole opening, a red message tells you. Check your wood and plates.

If your department tracks plates in Inventory, plates you have in stock are listed first.

## Reading the Results

A panel slides up with the answer. The title tells you how many struts fit, for example 6 struts fit. If nothing fits, it says No struts fit. The title counts the struts that fit the opening. How many struts the load needs is on each card.

The top of the panel shows the math: Raw opening, each deduction, then Required strut as a big number.

Under the math, Filter by system lets you tap LongShore, Acme thread or LockStroke to narrow the list. With none tapped, you see all three.

Each card shows:

•	The strut model, like LS 203. If it needs an extension, the card says so, like LS 203 + 24″ extension.

•	The system name.

•	The connectors you picked.

•	Rated load. This is the 4:1 working load from the Paratech chart row at or just above this length. It is the lower, safer number. LongShore has no published rating under 6 feet, so there the card shows the 6-foot figure. Acme thread and LockStroke under 2 feet show the 2-foot figure.

Cards marked Unrated come first. After them, the cards that need the fewest struts come first, then the cards with the fewest pieces.

Every card ends with: Planning aid, not an engineering certification.

To search again, close the panel and change a number.

The results sheet: the math, the system filter and the strut cards.

## Safety Warnings on Results

•	Unrated means LongShore above 16 feet (192 inches). The card says: LongShore above 16 ft (192″) is not rated by Paratech — rescue engineering consultation required.

•	Over capacity means the load is more than one strut can carry at the 4:1 safety factor. What to do depends on the number on the card.

•	×2 struts needed for this load or ×3: one strut is not enough. The card names the shore type that carries the load. Build a Double-T (2 struts) or a 3-Post (3 struts).

•	×4 struts needed for this load or more, for example ×5 or ×23: the card names no shore type. Plan more shore sets or call rescue engineering.

•	There is no cap at 4 struts. The 4:1 safety factor means a strut's rating is a quarter of the load that breaks it. Struts share the load equally, so a 100,000 lb load on struts rated 20,000 lb needs 5.

•	Fully extended — zero margin means the strut is at its longest reach. There is no room if the opening grows.

# 3. Operations

Operations is where a job gets tracked. One operation is one incident. Each spot you shore is a shore point. A shore point moves through seven steps, from Pending to Returned.

Starting, editing and ending an operation needs the Create & end operations permission. Admins have it. The Default role does not.

## Starting an Operation

1.	Tap Operations.

2.	Tap Start Operation.

3.	Type the Operation name, for example Cascade Building Fire.

4.	Choose Utilize Pending Card on or off. See the next section.

5.	Turn on Multi-building if the incident has more than one building.

6.	Optional: type the Location / address. If suggestions appear, tap one. Typing by hand always works.

7.	Tap Start Operation.

You become the Incident Commander. Only one operation can be active at a time.

To change the name, the deploy mode or the address later, tap the pencil next to the operation name.

The Start Operation form.

## One-Step or Two-Step Deploy

Utilize Pending Card sets how struts get assigned.

•	Off (the default) is one step. You find and deploy struts right in the Add Shore Point form. Best for small incidents.

•	On is two steps. You save the shore point as Pending. Someone assigns equipment from the board later. Best for big incidents where a separate group fetches struts.

You can change this any time with the pencil. Assign Equipment works in both modes.

## The Operations Board

The top of the board shows the operation name, when it started, the OP number, how many shore points and crews, and who is IC and what role you hold.

•	Tap Board or Cutting at the top. Cutting shows how many cuts are waiting.

•	Three small icons switch the layout: Board (a column for each step), List, and Division (by floor).

•	On a phone, the board groups the seven steps into Strut placement, Cutting, and Wood & return. On a tablet or laptop (768 pixels wide and up), each step has its own column.

•	Tap the filter button to open Filters & sort. Under Show, pick All shore points or Mine. You can also sort and filter by building, division, area and apparatus. Active filters show as chips. Tap Clear all to reset.

•	A step with nothing in it shows a zero. In one-step mode the Pending step reads skipped — one-step deploy.

•	Two side tabs open Available Inventory (gear on each rig) and Task Level Checklist.

•	At the bottom are Begin Briefing and End Operation.

The Operations board with an active operation.

## Adding a Shore Point

1.	Tap + Add Shore Point at the bottom right on a phone. On a tablet or laptop (768 pixels wide and up) it is + Shore Point at the top.

2.	If the operation has more than one building, tap Building. The first time, type the building name under Add building and tap + Add building. After that, pick from the list. The form stays off until a building is entered. It says Enter the building.

3.	Pick the Division. To add a floor, open Division and tap + Add floor above or + Add floor below.

4.	Optional: pick the building Side and type the Area / Room #. Side A faces the street. B, C and D go clockwise as you look down.

5.	Optional: tap Capture location (what3words) to save this phone's GPS position.

6.	Optional: pick Assigned (the rig working this shore) and type a Label.

7.	Pick the Shore type: T-Shore, Double-T or 3-Post. A T-Shore uses 1 strut, a Double-T uses 2 and a 3-Post uses 3. For a Double-T or 3-Post, a line under Number of Shore Sets shows the count, for example 1 Double-T = 2 struts.

8.	Set Number of Shore Sets.

9.	Enter the Measurement, open Deductions, and type an Estimated load. These work like Quick Find. Estimated load is the load one shore set must carry. FieldShore splits it evenly across that shore's struts. Do not type a per-post load. Do not type the total for several shore sets.

10.	In one-step mode, tap Find Available Struts, then Deploy on the strut you want. Or tap Save as Pending. In two-step mode, tap Add Shore Point.

Good to know:

•	Header and footer wood are required. Deductions opens by itself and shows Pick header and footer wood until both are picked. Only 4×4 and 6×6 are offered here. 3-Post fills in 6×6 for you. T-Shore and Double-T do not. Pick the wood you will really use: the Cutting Station cuts for it.

•	Each shore set is its own shore point. A Double-T or 3-Post makes one card for each strut, marked POST 1/2, POST 1/3 and so on.

•	If a Double-T or 3-Post set adds up to more than 10 struts, the form tells you to double-check the count.

•	Every shore point gets a number. A deleted number is never used again.

•	Location: the three words fill in when you have signal and the what3words service is working. Otherwise FieldShore saves only the coordinates.

•	If no stock is available, the form says No stock available to deploy — this will be added to Pending. The button then reads Add to Pending.

The Add Shore Point form.

## Assigning Equipment

Use this on a Pending card.

1.	Find the card under Pending.

2.	Tap Assign Equipment.

3.	The panel lists struts that fit and are on your rigs. Each says where it is stored, like Equipment located on: Rescue 2.

4.	Tap Deploy on the one you want.

5.	If the panel opens Review sources, check the truck for each piece. Fix any piece that is missing, then tap Confirm & deploy.

What you may see:

•	Deploying takes every piece (strut, plates and extensions) off the rig's available count right away.

•	A piece that is not on scene gives you three choices: Add to inventory, Deploy off-book (untracked), or Drop this plate (set to none). Off-book pieces do not come out of any stock count.

•	For an unrated strut, tap Team acknowledges the unrated zone before Deploy turns on.

•	If one strut cannot carry the load, the card offers Add 1 more strut — deploy as Double-T. Or tap Team acknowledges the over-capacity deploy, then Deploy 1 of 2 anyway.

•	If the load needs 4 or more struts, the card names no shore type. It tells you to plan more shore sets or call rescue engineering. You can still tap Team acknowledges the over-capacity deploy, then Deploy 1 of 5 anyway.

•	If nothing is on scene: No fitting strut is on scene. Deploy one off-book, or add it to a truck.

•	A Pending card tells you why it is waiting: Waiting for inventory or No matching strut. The message clears when stock changes.

The Assign Equipment sheet lists struts that fit and are on your rigs.

## Moving a Shore Point Through Its Steps

| Short name | Full name | What it means |
| --- | --- | --- |
| Pending | Pending Equipment | Measured. No equipment yet. |
| Assigned | Equipment Assigned | Equipment is pulled from the rigs. |
| Strut Set | Strut Set | The strut is in place. |
| Cutting | Cutting Station | The wood is being cut. |
| Runner | Runner | Cut wood is on its way to the shore. |
| Secured | Wood Shore Secured | The wood shore is in. |
| Returned | Strut Equipment Returned | Strut and plates are back on the rig. Final. |

On a touch screen you move a card with a slide. A slide keeps wet gloves from moving a card by accident.

1.	Put your finger on the bar at the bottom of the card.

2.	Drag it to the right, most of the way across.

3.	Let go. The card moves to the next step.

Each step has its own bar:

| On this step | Slide to move on | Slide back |
| --- | --- | --- |
| Pending | Tap Assign Equipment | none |
| Assigned | Slide to set Strut Set | Slide back to Pending Equipment |
| Strut Set | Slide to send to Cutting Station | Slide back to Equipment Assigned |
| Cutting | At the Cutting Station: Slide to mark Cut Done, then Slide to send to Runner | Slide back to Strut Set |
| Runner | Slide to set Wood Shore Secured | Slide back to Cutting Station |
| Secured | Tap Remove & Return Equipment | Slide back to Runner |
| Returned | none | none |

On the board, the Cutting step is read only. Work cuts at the Cutting Station.

With a mouse, each slide is a single button. Click it once. For example, Set Wood Shore Secured or Back to Cutting Station.

A shore point card at Strut Set with its two slide bars.

## Going Back a Step

Every slide has a matching slide back. Going back is normal. It does not need approval.

One slide back asks you to confirm. Sliding back from Equipment Assigned to Pending puts the strut, plates and extensions back on their rigs. A panel asks, for example, Return LS 203 to inventory?. Tap Return & Step Back. For a multi-strut shore the button reads Return All & Step Back.

The last step is different. Remove & Return Equipment is a button, not a slide. It asks you to confirm. It puts the gear back on the rig and moves the card to Returned. The panel says This cannot be undone. Returned is final.

## Multi-Strut Shores

A Double-T or 3-Post has one card for each strut. The cards stack in one pile on the board.

•	No strut can move to Strut Set until every strut of the shore has equipment. The slide shows Waiting on group — 1 of 3 still Pending.

•	From Equipment Assigned through the Cutting Station, the struts move together. This works going back too.

•	From Runner on, each strut moves on its own.

•	If other struts of the shore already have equipment and you change the measurement or deductions, a panel asks Re-measure this shore?. Update all legs changes every strut and flags the struts with equipment for a re-check.

•	Deleting one strut of a shore deletes the whole shore.

## The Cutting Station

The Cutting Station is for the person on the saw. It lists every shore point at the Cutting step, oldest first. The length to cut is a big number.

1.	On Operations, tap Cutting at the top.

2.	Read the top card, Cut this now. It shows the Cut length and where the piece goes.

3.	Cut the wood to that length.

4.	Slide Slide to mark Cut Done. The card shows Cut done.

5.	Slide Slide to send to Runner. The card leaves your queue and moves under Sent to runner.

The math is on the card: the opening, minus the header and footer wood, minus a 1½ inch wedge. It rounds down to the nearest eighth of an inch.

The wood in the math is the header and footer wood picked when the shore point was added. A 6×6 is 5½ inches thick. A 4×4 is 3½ inches. If the header and footer differ, the math names each one. A shore point saved before wood was required, with no wood picked, uses the standard for its shore type (4×4 for a T-Shore or Double-T, 6×6 for a 3-Post) and says (standard, none picked).

More about the station:

•	Made a mistake? Slide Slide back — clear Cut Done. Or slide Slide back to Strut Set.

•	A warning shows if the cut length would be zero or less: Opening too small for this header and footer wood — verify measurement. Re-measure the opening.

•	The list under the top card shows waiting cuts in order. Cuts cannot be re-ordered.

•	For a second saw, tap + Add saw. Each saw gets its own Cut this now card. A free saw takes the top waiting cut. On a phone with more than one saw, pick your saw first.

•	A badge like 2 new means cuts were sent from another device.

The Cutting Station with the cut length as a big number.

## Shore Point Details

Tap a card once it has equipment to open its details. You see:

•	Where it is and which step it is on.

•	Required strut length, the big number.

•	Measurement & load: the raw opening, each deduction, and the load.

•	Location: the what3words address and the coordinates, if captured.

•	Bill of materials: every piece and the rig it came from. Off-book pieces say so.

•	Timeline: every change, when it happened, and whether this device or another device did it.

•	Safety: whether the strut is within its rating, or a warning.

A change that lost a race shows — no effect in the timeline.

## Editing, Deleting and Restoring a Shore Point

•	Edit and Delete show only on Pending cards. Tap the card's header to see them.

•	You can edit only a Pending card. If other struts of the same shore already have equipment, changing the measurement or deductions asks Re-measure this shore?. Update all legs changes every strut and flags the ones with equipment for a re-check.

•	Capture location shows on any card that has no location yet, until it is Returned. To capture again, edit the card while it is Pending.

•	Delete asks you to confirm. A shore that holds equipment cannot be deleted. Return the equipment first.

•	A deleted shore point moves to Deleted at the bottom of the board. Open it and tap Restore. The shore point comes back with its old number.

## Ending an Operation

You need the Create & end operations permission.

1.	Tap End Operation. It is at the bottom of the Operations board and at the bottom of Command.

2.	Read the warning. If shore points still hold equipment, it lists them by rig, for example 2 shore points still hold equipment — Rescue 2 (2).

3.	If all that gear is really back on the rigs, tick All equipment is back on the rigs.

4.	Tap End Operation.

Ending an operation archives every shore point. You can start a new operation after.

If you leave the box empty, those units stay counted as deployed. Inventory does not show gear that is still out as if it were home.

The End Operation confirm lists equipment still out.

## Reopening a Past Operation

When no operation is active, Past operations lists the finished ones.

1.	Tap a past operation to look through it. A banner says Viewing a closed incident — read-only.

2.	Tap Re-open this incident.

3.	Tap Re-open.

The operation becomes active again with every shore point as you left it. You can only re-open when no other operation is active.

## Risk Briefing and Task Level Checklist

•	Begin Briefing opens the Risk briefing (ORM / TCRM) checklist. It covers the plan, the hazards, crew input, and the rule that anyone can stop the operation. It has five crew questions too. Tap End Briefing when done. If a briefing is running, the button reads Open Briefing.

•	The briefing records that it happened. It never blocks crew entry, deployment or any status move.

•	The Task Level Checklist side tab holds five sections: Assessment, Search, Access, Extricate and All-clear.

•	The built-in checklist wording is placeholder text, not approved doctrine. Before field use, your Admin must replace it with your department's wording. See Custom ICS Titles, Checklists and Policies.

# 4. Command

Command shows who is in charge and what is happening across the whole incident. It needs an active operation. Without one, it says No active operation. Tap Go to Operations.

## The Command Screen

From top to bottom on a phone:

•	The operation name, its address, and the OP clock with elapsed time.

•	Counts of shore points, apparatus and individuals. A gold pending sync count shows when role changes have not reached the cloud yet.

•	A hazard chip when any hazard is open.

•	The command staff card: Incident Commander, Operations Section Chief and Safety Officer.

•	Shore points by step. Switch between All incident and By Division.

•	Rows for Hazards and Org Chart.

•	The Resource roster.

•	End Operation.

If you are the IC, a side tab named IC Command Checklist is also on this screen.

The Command screen on a phone.

## The Org Chart

The org chart is the ICS structure for this operation. A new operation starts with these positions:

Incident Commander
├─ Safety Officer
└─ Operations Section Chief
   ├─ Rescue Group Supervisor
   ├─ Shoring Group Supervisor
   ├─ Staging Area Manager
   └─ Cutting Station

•	Tap a position to open it. Everyone can look. Only the IC can change the chart.

•	The Cutting Station position shows how many cuts are in the queue.

•	A position panel warns when a supervisor has too many direct reports.

•	Tap the full screen button to see the whole chart. Pinch to zoom.

•	The IC can drag a card to move it, or drag a rig onto a card to assign it. The Move… buttons in the position panel do the same without dragging.

The ICS org chart with the default positions.

## Assigning People and Rigs to a Position

Only the IC can do this.

1.	On Command, open Org Chart.

2.	Tap the position.

3.	Tap Change or add apparatus.

4.	Tap a rig under Apparatus on roster, or a person under Department members. Or type a name in Add individual and tap Add.

5.	Tap Back.

The first one assigned is the leader. Tap an assigned name again to take it off.

A person picked from Department members is tied to their account. They keep the position on any phone they sign in on. A name you type is just text.

## Changing the Chart

The IC can change the chart from a position's panel:

•	Rename a position.

•	Add position under this adds from the ICS list: command staff, deputy IC, section chiefs, branches, divisions, groups, strike teams, task forces, single resources and units.

•	Move… moves a position left or right, or under a different position.

•	Remove position asks you to confirm. It also removes everything under it. The built-in positions cannot be removed.

## Your Role and the Incident Commander

•	Tap My role on the Org Chart screen and pick the ICS position you are filling. Anyone can do this. Tap Clear my role to remove it. Your role sets the Mine filter on the Operations board.

•	The person who starts the operation is the IC.

•	Your role, and being IC, follow your account. If you sign in on a different phone, you are still IC there.

## Handing Over Command

Only the IC sees Transfer.

1.	On Command, tap Transfer next to Incident Commander.

2.	Pick who gets command. Choose someone on the org chart, an apparatus, a department member, or type a name under Transfer to someone new.

3.	Tap Hand over command.

4.	For a person you typed or a rig, the screen shows a 4-digit Accept code. Give it to them with your briefing.

5.	They tap Transfer pending (it says Tap if this is you), type the code, then tap Accept command.

Or hand them your phone and tap Accept command with their name on it.

A department member picked by name needs no code. They accept from their own account.

Command has not moved until the transfer is accepted. You stay IC until then. You can tap Cancel transfer. They can tap Decline.

## Operational Periods

The OP block at the top of Command shows the period number and elapsed time. It turns amber near the end of a planned period. It says Due now past the end.

1.	Tap the OP block.

2.	Pick a planned length: 12 h, 8 h or Custom.

3.	Optional: type an IAP / ICS-202 reference.

4.	Tap Start OP 2. The number is the next period.

At the end of a planned period, a card asks OP 1 complete — start OP 2?. Tap Not yet to wait.

## Hazard Log

The Hazard Log lists dangers on the scene. Anyone can add one.

1.	Open the Hazard Log. On a phone, tap Hazards on Command. On a laptop, tap Hazard Log above the right pane.

2.	Tap Add Hazard.

3.	Pick the Type: Structural instability, Utility, Atmospheric, Fall or Other.

4.	Type the Location. A bare Division number, like 2, also flags that Division's shore point cards.

5.	Pick the Severity: Low, Medium or High.

6.	Optional: add Notes.

7.	Tap Add Hazard.

Open hazards show first, highest severity first. Mitigated hazards are in their own list.

Tap a hazard to open it. Tap Mitigate hazard when it is dealt with. Tap Reopen hazard to undo.

Export ICS-208 saves the log as a CSV file.

The Hazard Log with open and mitigated hazards.

## IC Command Checklist

If you are the IC, a side tab named IC Command Checklist opens a four-phase checklist: size-up, initial actions, sustained operations and demobilization. Tick items as you go.

# 5. Inventory

Inventory counts struts, extensions and base plates on each rig. Everyone in the department can see it. Changing it needs the Manage inventory & apparatus permission.

## The Inventory Screen

•	The top shows your department name and how many apparatus you have. It has a data button (two arrows) and Add equipment.

•	Under it are totals: on hand, deployed and models.

•	One tab for each rig. The + tab adds a new rig.

•	Cards for Struts, Extensions and Base plates.

•	Each row shows what is available out of the total, like 3/6. A bar shows how much is still available. It shrinks as stock is deployed.

•	A chip says running low, 2 deployed, or all 6 deployed.

•	Over-allocated — needs resolving means two devices took the last unit while offline. Count the gear and fix the number.

Available is worked out for you. It is what is on the rig and not deployed on a shore point.

The Inventory screen for one rig.

## Adding an Apparatus

1.	Tap the + beside the rig tabs. On an empty Inventory, tap Add apparatus.

2.	Type the Apparatus name, like Engine 1.

3.	Pick the Type: Chief, Engine, Ladder, Rescue, Squad, Other, or a type your Admin added.

4.	Tap Add.

## Adding Equipment

1.	Tap the rig's tab.

2.	Tap Add equipment.

3.	In Add equipment for that rig, tap a strut, an extension or a connector plate to add one. Tap it again to add another. The number on the button shows how many are on the rig.

4.	Close the panel.

## Changing Counts

•	Tap + to add one. Tap − to take one away.

•	− is off when no unit is available. You cannot go lower than what is deployed.

•	Take the last unit away and the line is removed.

•	A member without the permission sees the counts but not the + and − buttons.

## Removing an Apparatus

1.	Pick the rig's tab.

2.	Scroll to the bottom and tap the button with the rig name, like Remove Rescue 2.

3.	If the rig has gear, a panel asks you to confirm. It says how many items and units stocked on the rig will be removed from inventory, for example 3 items (4 units) stocked on Rescue 2 will be removed from inventory. This cannot be undone. Tap the red button. It carries the rig's name, like Remove Rescue 2.

An empty rig is removed at once. The button is off while any gear from that rig is deployed. It says Return deployed equipment first.

## Importing Inventory from a CSV File

Import loads a whole department's gear at once, or restores a backup. You need the Manage inventory & apparatus permission. You can import during an operation.

Import accepts CSV files only. The file picker shows only .csv files. Excel files (.xlsx) and the Reference sheet are not supported in 4.0. Save your spreadsheet as CSV first.

The import opens in a full-screen window. There is no laptop side-by-side.

1.	Tap the data button at the top of Inventory, then tap Import inventory. You can also use Settings, then Data management.

2.	Step 1 of 4, Pick your file: tap Choose CSV file. The screen shows the file name, the columns and rows, and First 5 rows. Tap Change to pick another file. Tap Continue.

3.	Step 2 of 4, Map your columns: check that each column in your file matches a FieldShore field. Fix any that need it. Tap Continue.

4.	Step 3 of 4, Check the rows: read the counts and any problem rows. Tap Continue.

5.	Step 4 of 4, Review and import: fix or skip each flagged row. Tap the Import button. It shows the number of rows, like Import 12 rows.

6.	Read the result. Tap Done.

## Pick Your File and Map Your Columns

•	If the file has an unmatched quote ("), you get Couldn't read that file. Fix the quotes and pick the file again. Nothing is read.

•	If the file has only a header row, you get That file has no data rows.

•	Continue stays off until a file is picked.

•	FieldShore matches columns by name. It ignores capital letters. A matched column shows a check mark. A column with no match shows a warning and needs a pick.

•	For each file column, pick one FieldShore field, or Ignore this column. A field can feed only one column. Picking it again frees the old column.

•	Type, Apparatus and Quantity must each be matched. Continue stays off until they are.

•	If you turned on Native controls, the pickers are your phone's own.

Step 2 of the import: match each file column to a FieldShore field.

## Check the Rows and Fix Problems

Step 3 shows how many rows will import and how many need a fix. A blank row is ignored. A row with the wrong number of columns is flagged and can only be skipped. Each flagged row says why, for example Row 7: unknown strut Model "LS 999" — skipped.

FieldShore checks each row in this order. The first problem it finds is the one it shows:

•	Type is Strut, Extension or Plate.

•	Rows that say EXAMPLE — delete before importing are ignored. Those are the template's example rows.

•	Apparatus is filled in.

•	Quantity is a whole number above zero. Decimals and forms like 1e2 or +3 are not accepted.

•	A strut's Model is in the Paratech catalog.

•	An extension's length is a whole number, and its System is Gold, Grey or LockStroke. LongShore and AcmeThread also work if spelled exactly like that.

•	A plate's Plate ID is in the catalog.

A model or plate that is not in the catalog is flagged and not imported.

About the System column: for struts, the system comes from the catalog, so the column is not needed. For extensions it is required. Use Gold, Grey or LockStroke. The words LongShore and AcmeThread also work. Capital letters must match exactly in this column. Gray also works. A System typed as Acme thread or gold is flagged. The Plate Name column is never read. Plate ID is used.

If an operation is active, a note says deployed items are not changed, and any row that would strand deployed gear is skipped.

In Step 4, each flagged row has a card with the problem and a field to fix it. The field depends on the problem: a strut model picker, a plate picker, a quantity box, a system picker, a type picker or an apparatus box. After a fix, that row is checked again. Tap Skip this row instead to skip it, or Undo to bring it back. Only valid rows that you did not skip are imported.

If you go back to Step 2 and forward again, your fixes and skips reset.

## What Import Does to Your Counts

•	All or nothing: the whole import saves together. If it fails, nothing is saved.

•	A row with an ID sets the count for that item. A row with no ID is matched to the same item on the same rig. If nothing matches, it makes a new item.

•	Rows with no ID for the same item inside one file add up. Two rows of 2 and 3 become 5.

•	The total for an item in the file replaces the count you have now. Importing the same file twice does not double your stock.

•	A new Apparatus name makes a new rig with the type Other.

•	Import never deletes. Items that are not in the file stay as they are.

•	A row is skipped when its quantity is below the number now deployed. This protects deployed gear. There is no pop-up. The result counts it.

•	A row is skipped when its ID matches an item of a different kind.

## Import Results

While the import saves, the screen says Importing…. You cannot close it.

When it finishes you see Import complete, then lines such as:

•	12 rows imported.

•	2 rows merged with identical rows.

•	1 skipped — would strand deployed stock.

•	1 skipped — type doesn't match the item already on the rig.

•	1 row not imported — still flagged to fix.

•	1 row skipped by you.

•	1 blank row ignored.

The last line says where it saved: Saved on this device and in the cloud. With no signal it says Saved on this device — will sync to the cloud when you reconnect. If some rows did not reach the cloud, an amber line says so.

If the save fails, you see Import failed and Nothing was saved.

## Exporting Inventory and the Template

You need the Export / delete data permission.

•	Export inventory saves a file called fieldshore-inventory.csv. It has ten columns. Items with a zero count are left out. It is sorted by apparatus, then struts, extensions and plates.

•	Download template saves fieldshore-inventory-template.csv. It has the same ten columns and three example rows: one strut, one extension and one plate. The Apparatus column on each example says EXAMPLE — delete before importing. Import ignores rows with exactly that text.

•	Find both under the data button on Inventory, or on Settings, then Data management.

•	In the Apparatus and Model columns, text that starts with =, +, - or @ is saved with a leading apostrophe. This stops a spreadsheet from running it as a formula. Import takes the apostrophe off again.

A member with neither permission sees no data buttons.

# 6. Settings

## Settings Pages

On a phone, Settings opens a list. The theme picker is right at the top. On a laptop, the pages sit under Settings in the left rail.

| Page | Who sees it |
| --- | --- |
| Account | Everyone |
| Appearance | Everyone |
| Department | Everyone |
| Data management | Members who can manage inventory or export data |
| Apparatus types | Members who can manage department settings |
| Administration | Members who can manage users or settings, and the IC or Operations Section Chief |
| About | Everyone |
| Help & reference | Everyone |

The Settings list with the theme picker.

## Account

•	Your name comes from sign-in. You cannot change it here.

•	Type your Rank / title and tap Save. It shows next to your name on the roster and in the audit log.

•	Your email is read only.

•	Log out returns you to guest mode. Your work on the phone stays put.

•	Delete account asks for your password and cannot be undone. It removes you from your department's roster. The department stays for your crew, and the work you logged stays in its record. It also erases FieldShore's data from this phone, including changes that have not synced. Do it with signal, once the sync bar is gone. If you are the only Admin, make someone else an Admin first.

## Themes

1.	Tap Settings. On a laptop, tap Appearance in the rail.

2.	Tap System, Light, Dark or Sunlight.

Dark is the default. System follows your phone. Sunlight is for direct sun on scene. It uses thicker lines and black on white.

Tap More appearance settings for Native controls. It makes the app use your phone's own pickers instead of FieldShore's. This helps screen readers.

## Department

This page shows your department name and an Admin badge if you are one. It also shows the invite code this phone last saw, with its QR code. The person who created the department has it. A member who joined by code has the code they used. If your Admin has used Regenerate, that code no longer works. Get the new one from your Admin.

If you have no department yet, tapping Settings shows Set up your department. It has Create department, Join an existing department and Sign out.

## Users and Roles

You need the Manage users & roles permission. Open Settings, then Administration, then Users & roles. The screen is called User manager. It has Members and Roles.

To add a member:

1.	Open User manager.

2.	Tap + Add member.

3.	Type their Name and their Email. The email is their sign-in.

4.	Pick their Role and Apparatus. Rank, badge, phone and certifications are optional.

5.	Tap Create account.

6.	The panel Account created shows their email and a starter password. Read it to them in person. It is not shown again.

7.	Tap Done.

More you can do:

•	A key badge that says starter means the member has not set their own password yet.

•	Tap a member to change their role, edit their profile, or change their email. Changing an email signs them out everywhere.

•	Reset password gives them a new starter password and signs them out on every device.

•	Revoke access takes access away at once. Their record stays so the activity log is readable. Tap Reactivate to bring them back.

•	The last Admin cannot be revoked or demoted.

•	Import CSV adds many members at once. Columns are Name, Email, Rank, Apparatus, Badge, Phone, Certs and Role. Each row becomes a login with a starter password. The window has a blank template to download.

•	The invite code is on the Members tab. Regenerate stops the old code working at once. Other phones keep showing the old code. Tell your crew to get the new one from you.

On the Roles tab, Admin is built in and fixed. Default is built in. Its name is fixed, but its permissions can change. Only an Admin can tap + Create role or edit a role. Deleting a role moves its members to Default.

The User manager Members view.

## Roles and Permissions

A role is a name plus a list of things it can do. There are eight switches:

| Permission | What it unlocks |
| --- | --- |
| Read department data | No 4.0 screen or rule checks it yet. Every active member can see the department's data. |
| Run field work | Part of the Default role. No 4.0 screen checks it yet. |
| Create & end operations | Start, edit and end an operation. |
| Manage inventory & apparatus | Change counts, add and remove rigs, import. |
| Manage roster | No 4.0 screen checks it yet. |
| Manage department settings | Apparatus types, custom ICS titles, checklists, policies. |
| Manage users & roles | User manager. |
| Export / delete data | Export inventory and download the template. |

Admin has all eight. Default has Read department data and Run field work.

Some things depend on your place in the incident, not your role. Only the IC can change the org chart, hand over command, and see the IC Command Checklist.

## Audit Log

The audit log records who did what. Open Settings, then Administration, then Audit log.

•	Incident shows the active operation, or the last one that ended. The IC and the Operations Section Chief can see it.

•	Administrative shows the department record. Members with Manage users & roles can see it.

•	Type in the filter box to search. Scope groups by action, by person, or (Incident only) by period.

•	Tap CSV to save the view you are looking at.

•	Without access you see Command record.

## Apparatus Types

You need the Manage department settings permission. Six types are built in: Chief, Engine, Ladder, Rescue, Squad and Other. To add your own, such as Water Tender, type it under Add a type and tap Add type. You can remove types you added. A type that a rig still uses cannot be removed. The message Type is in use lists those rigs.

## Custom ICS Titles, Checklists and Policies

You need the Manage department settings permission. These are on Administration.

•	Add custom title adds a role to the ICS list. Strike Teams and Task Forces can carry a crew make-up that fills in when you place them.

•	Checklists let you change the IC Command Checklist, the Task Level Checklist and the Risk briefing. You can edit, add, remove and re-order steps and sections. Changes apply to your whole department. Reset to FieldShore baseline puts the original back. The built-in wording is placeholder text, not approved doctrine. Before field use, replace it with your department's wording.

•	Send after-action record by email saves your preference. Email delivery is not on in 4.0.

## Help and Reference

•	Replay the tour runs the intro again.

•	Quick Find walkthrough shows tips on the real Quick Find screen. Nothing you tap is saved.

•	Field — measure and shore and Command — IC and org save the kind of guided help you want. In 4.0 they do not change the tours yet.

•	Open user guide opens this manual. It works with no signal.

•	Send feedback sends a Bug, Idea or Other note. It goes with your app version and department name.

•	Reference links open the USACE Shoring Operations Guide and the FEMA US&R Response System overview.

## About and Updates

About shows the version, like Version 4.0.0. FieldShore updates itself in the background. Tap Check for updates to look right now. If it finds one, the app reloads. See what FieldShore does opens the welcome page.

# 7. Working Offline

## What Works With No Signal

FieldShore saves on your phone first. With no signal you can still use Quick Find, add and move shore points, deploy, run the Cutting Station, log hazards, use Command, and change or import Inventory.

You need signal for:

•	Creating an account and signing in.

•	Joining a department. FieldShore holds the code and finishes the join when signal returns.

•	The three words of a what3words address, when the what3words service is working. Coordinates are saved in the meantime.

•	Address suggestions. Typing an address by hand always works.

•	Sending feedback.

•	Loading the member list and the Administrative audit log.

•	Adding a member, resetting a password and changing a member's email.

•	Set your password.

•	Delete account.

•	The reference links in Help & reference.

## The Sync Banner

Signed-in members see a bar under the header when something needs attention. When everything is saved to the cloud, the bar is gone.

| The bar says | What it means |
| --- | --- |
| Offline — 3 changes saved on this device — will sync when you reconnect | No signal. Your work is safe on this phone. |
| Syncing 2 changes… | Signal is back. Changes are going up. |
| 2 changes haven't synced yet — retrying | Uploads are stuck. FieldShore tries again on its own. |
| Will join "name" when you reconnect | A join is waiting for signal. |
| "name" is saved on this device — will sync when you reconnect | Your new department has not reached the cloud yet. With signal it reads "syncing to the cloud (invite code works after that)". |
| Synced — 1 of your changes had no effect | Another device got there first. Tap to see. Tap Got it to clear. |

Guests do not see a banner.

The sync bar while offline.

## When Signal Returns

FieldShore sends your saved changes in the order you made them. It tries again on its own, after 5 seconds, then 15, then 60. You do not need to do anything.

For inventory counts, rigs, custom titles and checklists, the newest change wins.

## When Two People Change the Same Thing

Every phone puts the changes in the same order: the order the cloud received them. If two people act on the same shore point, the first change to arrive wins. The other change has no effect.

When that happens, the sync bar says Synced — 1 of your changes had no effect. Tap it. Each line tells you what happened, for example: #3 · West Window — now Cutting Station. Your Strut Set had no effect. Another device moved it to Cutting Station while you were offline. Check the board and do it again if you still need to.

If the department server refuses a change, the line reads Rejected by the department server — not shared with the team.

If two phones took the last unit of a strut while offline, Inventory shows over-allocated — needs resolving for that line. Count the gear and fix the number.

# 8. Safety Notes

•	FieldShore is a planning aid. It is not an engineering certification. Every result card says so.

•	Lengths round down to the nearest eighth of an inch. FieldShore never rounds up. A cut is short on purpose. The wedge takes up the slack. A cut that is too long is the hazard.

•	The cut length uses the header and footer wood picked when the shore point was added. Pick the wood you will really use.

•	LongShore above 16 feet (192 inches) is not rated by Paratech. FieldShore warns you and asks for acknowledgment.

•	FieldShore has no cap on struts for one opening. The card shows how many the load needs.

•	A deploy that is short of the struts the load needs requires a recorded team acknowledgment.

•	Unknown connector on a card means the connector is not in this app's catalog. Its height is missing from the length, so the length shown is too long. Do not set or cut to it. Update the app or re-check the connectors.

•	A card marked Opening too small — verify measurement has no usable cut length. Re-measure.

•	The Risk briefing checklist never blocks work. It records that you held the briefing.

•	The built-in checklist wording is placeholder text, not approved doctrine. Before field use, your Admin must replace it with your department's wording. Go to Settings, then Administration, then Checklists.

# 9. What Is Not in 4.0

•	Excel files (.xlsx) and the Reference sheet for inventory. Use CSV.

•	Re-ordering the Cutting Station queue.

•	Sending the after-action record by email. The setting saves, but no email goes out yet.

•	A map of the operation address.

•	A TV display (Broadcast) theme. Settings offers System, Light, Dark and Sunlight.

•	Switching departments, leaving a department, or deleting a department inside the app. An account uses one department. Ask the FieldShore admin to remove a department.

•	A button to remove one line of inventory. Take the count down to zero with −.

•	A separate form for adding visiting-department equipment.

•	Mutual-aid invites between departments.

## If You Used FieldShore 3

| In version 3 | In 4.0 |
| --- | --- |
| Sign in with a Department ID | Sign in with email and password. Join a department with an invite code. |
| Strut Installed | Strut Set |
| Cut Table | Cutting Station |
| Gold and Grey systems | LongShore and Acme thread. LockStroke is the same. |
| Hold the Quick Start button to start an operation | Start Operation on the Operations tab |
| Fractions to 1/16 inch | Fractions to 1/8 inch |
| Excel import | CSV import |
| Mobile and desktop toggle button | The layout follows the screen width |

# 10. Version History

Major and minor releases only. Patch releases (bug fixes) are left out. Versions before 4.0 are the earlier FieldShore (version 3).

| Version | Date | Highlights |
| --- | --- | --- |
| v4.0 | At release | A new FieldShore, rebuilt for crews who work with and without signal. Accounts and departments: sign in with email and password, create a department, join with an invite code or QR code, and let an Admin add members with a starter password. Quick Find: works for everyone, lists unrated warnings first, and rounds lengths down to 1/8 inch. Operations: start an operation, add shore points, assign equipment, and move shore points with slides. Every slide has a slide back. One back-step and the final return ask you to confirm. Cutting Station with more than one saw. Command: org chart, command hand-over with an accept code, operational periods and a Hazard Log. Roles follow your account, not your phone. Inventory: per-rig counts, a four-step CSV import with fixes and skips, and CSV export. Settings: themes including Sunlight, roles and permissions, audit log, checklists. Working offline: everything saves on the phone first. A sync bar tells you what is waiting, and tells you when another device's change beat yours. The app opens straight to Quick Find. The welcome page shows on a first browser visit, and an installed app skips it. |
| v3.22 | 2026-06-07 | Inventory-integrity & measurement-safety release. Measurements now round _down_ to 1/8" (cut and set-to lengths) so you are never told to cut long — short is taken up by the wedge (issue #300). Over-allocation safety flag: if two crews deploy the same strut at once while offline, on reconnect the app flags the shore point with a persistent VERIFY PHYSICAL STRUT badge (cleared only by IC / Safety) instead of silently losing count, and a flagged point is not counted as secured (issue #80). Under the hood: stronger validation on visiting-department equipment counts, and a clean-up of the inventory deploy/return code with no change in behavior (issue #284). _(Header/footer beams and the Command department breadcrumb were planned for this release but deferred.)_ |
| v3.21 | 2026-05-25 | All measurements now display as fractions (issue #119, Hartsdale field feedback). Shore-card lengths, deduction summaries, cut table expected/actual, archived ops, Quick Find results, and Quick View inventory all render in tape-measure format — 48-1/2" instead of 48.5" — at 1/16" precision. The "Actual cut" override on the cut table is now a feet/inches/fraction picker matching the rest of the app, so cutters don't have to mentally convert tape-measure readings into decimals. Partially-typed actual measurements survive Firebase status pushes from peer devices. Excel import now accepts fractional text like 48 1/2 in the Extension Length column (previously silently truncated to 48). Excel exports remain in decimal for spreadsheet math compatibility. |
| v3.20 | 2026-05-25 | External equipment now flows into "available inventory" end-to-end (issue #127). Adding external struts, extensions, or plates on the Command tab now makes them deployable on shore points — previously external extensions were silently invisible to the deploy modal, and external plates had a pre-existing silent loss on return. External items also now appear in the Quick View "Available Inventory" sidebar in a dedicated External Department Equipment section, with the source dept and apparatus shown on a subordinate sub-label under each item. The Inventory tab is unchanged — it remains dept-owned-only by design (visiting-dept gear is op-scoped and lives on Command + Quick View). |
| v3.19 | 2026-05-20 | Default ICS org chart restructured for NIMS Type IV–V incidents. Added two new default roles: Staging Area Manager (under Operations per NIMS doctrine) and Division 1 (under Operations, collapsed by default to fit phone screens). The four tactical roles — Entry, Rescue, Initial Shoring, Wood Shoring — now sit under Division 1 instead of directly under Operations. Cutting Table and Runner stay where they were. Card titles (v3.18.2 fix carried forward) accept up to 30 characters with word-wrap and no subtitle row. |
| v3.18 | 2026-05-20 | hfd217 field-feedback response. Removed: scenario presets in Start Operation (added cognitive load at incident-start) and the Solo-IC / Auto-promote mode block on Command (exposed implementation state as user concept). The Command tab now always renders the full view regardless of apparatus count. Added: three dashboard tiles at the top of Command — Elapsed (HH:MM:SS, ticks every second), Apparatus count, Shore Points count. Fixed: cutting table Send to Runner action restored (workflow blocker). Operations Section Chief role can now mark cut-done and send to runner (battalion-chief doctrine — Ops keeps physical-custody actions off the Command tab). Adding external equipment now updates the available inventory display immediately on both the Command and Inventory tabs. Operations tab header no longer shifts left when the inventory right-rail is on. Assigned Apparatus column now groups chips under per-category labels (Chief, Deputy Chief, Engine, Ladder, etc.) instead of running labels inline. Plate picker sorts in-stock plates to the top with "Available" / "Not in inventory" section labels. |
| v3.17 | 2026-05-19 | "Strut Placed" renamed to "Strut Installed" (FEMA US&R FOG) everywhere in the UI — badges, buttons, legend, drilldown counts. Quick Start FAB on the Quick Find tab: press and hold 500ms to instantly create an operation and jump to the first shore point, skipping the Start Operation modal entirely. End Operation moved to the Command tab header — IC only. Send to Runner now requires the Runner role (plus IC override). Mark Secured now requires IC, Safety, or a Shoring role (Initial Shoring, Wood Shoring). Add External Equipment form rebuilt to match the regular inventory-add layout — struts, extensions, and connector plates in one quick-add grid with Source Department and Source Apparatus fields at top. External equipment returns now reconcile automatically after offline use or connection failures. _(Scenario presets and Solo-IC mode shipped in v3.17 but were removed in v3.18 after field feedback — see v3.18 row.)_ |
| v3.16 | 2026-05-19 | SmartArt-style ICS org chart. The command hierarchy now renders as a visual tree with L-shaped connector lines between parent and child roles. Each card has an inline toolbar with reorder controls (Up, Down, Promote, Demote) so you can restructure the hierarchy without drag-and-drop. On phones the toolbar is hidden and the same controls appear in the node-tap modal. Only the IC can add, remove, or rearrange roles (changed from IC+Safety per NIMS SM-0322). Role assignment now shows a confirmation toast. All org chart buttons meet 44px minimum touch targets for gloved use. |
| v3.15 | 2026-05-19 | Numbered divisions with vertical anchoring. Division is now a top-to-bottom picker — Div 3 / Div 2 / Div 1 (Ground) / Sub Div 1 (Basement) — with + Floor Above and + Sub Div Below buttons. No manual text entry; the division list is shared across the operation so labels stay consistent. Existing operations migrate automatically (numeric and 'Ground'/'Basement' labels convert in place; anything else is preserved as a legacy label with a non-blocking review banner). Offline inventory hardening: deploys and returns made while offline now reconcile correctly with Firebase on reconnect via a per-item touched-set flush-pass (closes the v3.8.2 architectural root cause). Concurrent-deploy race guard — when another device takes the last unit, the second device sees "Another user took the last X — try again" and the orphaned shore-point write is rejected. |
| v3.14 | 2026-05-18 | Operations tab on desktop now splits into two columns. Left sidebar (sticky) shows a drilldown tree of Buildings → Divisions → Areas with live status counts on each node, and a search input above the tree filters by label / building / division / area. Right column shows the filtered shore points with the + Shore Point button centered above the status groups. Mobile is unchanged (inline breadcrumb stays). |
| v3.13 | 2026-05-18 | Desktop view. On screens 1024px wide and up the app now uses a horizontal top nav bar and widens the container to 1200px. The Command tab splits into two columns on desktop — roster (Apparatus, External, Individuals, My Role, Hazards) on the left (sticky), Dashboard / ICS Organization / Layout on the right. A small toggle in the upper-right of the viewport flips between desktop view and mobile view; the choice is remembered. Phones and iPad portrait are unchanged. |
| v3.12 | 2026-05-18 | Command tab promoted to top-level navigation (apparatus, individuals, org chart, hazard log separated from shore-point workflow). Hazard Log (ICS-208 style) with severity tiers, mitigate/reopen, and archived op persistence. End Operation now requires typing "END" to confirm (glove-safe gate). Force-update overlay for critical fixes (data preserved). Yellow span-of-control warning tier at 6–7 direct reports. Offline banner shows queued write count. Settings → Check for Updates button. |
| v3.11 | 2026-05-17 | Field-feedback round-up. Quick Find plate picker selection fixed (tap on a connector now actually selects it). Header scrolls away with content instead of locking real estate at the top. Edit Apparatus modal no longer shows the Add form simultaneously. Quick-add inventory list preserves scroll position when adding items near the bottom. Pending shore-point length edits now keep effective length in sync when deductions are toggled off, with success toast. Comprehensive dark-mode contrast pass — buttons, plate pickers, apparatus tabs, qty controls, and small action buttons are all theme-aware (no more black-on-dark text). |
| v3.10 | 2026-05-16 | LongShore unrated-zone deploys now require an explicit acknowledgement modal (audit trail captures who/when). Pending shore points have a real "Assign Equipment" deploy flow. Locked-card status buttons are role-gated (Cutting / Runner / Entry-Rescue-Shoring, with IC / Safety override). Excel imports warn before orphaning deployed equipment references. Persistent banner on auth-failure / sync degradation with manual retry. SheetJS bundled into the service worker for offline import/export. Fully-extended boundary warning on strut results. |
| v3.9 | 2026-05-16 | Group status guard — sending one shore point back through the workflow no longer drags advanced group-mates with it. Excel imports support extensions and connector plates (not just struts). Firebase scripts pinned with Subresource Integrity. Stored XSS hardening on shore point card rendering. |
| v3.8 | 2026-05-16 | Individual wood cut tracking per shore point card (grouped points no longer share cut/runner/secured lifecycle). Inventory tab updates immediately after deploying or returning equipment. Sync diagnostics for offline write queue troubleshooting. |
| v3.7 | 2026-05-15 | Firebase Anonymous Auth + security rules. Photo attachment in feedback form. Status dot key/legend on ICS org chart. Liability disclaimer (removed). Clearer empty state when inventory lacks a fitting strut. Conservative-floor interpolation for load capacity. |
| v3.6 | 2026-05-15 | Firebase listener cleanup (no more leak on department switch). Concurrent edit safety for org chart swaps. Keyboard accessibility for all interactive elements. Performance optimization for apparatus name lookups. |
| v3.5 | 2026-05-14 | Dark/light/system theme toggle. Shore point Group field changed to apparatus dropdown. Local-first write architecture across all mutation sites. |
| v3.4 | 2026-05-14 | Command page overhaul: role reparenting (long-press drag + "Move to..." menu), collapsible branches, status indicators (active/staged), headcount badge, span-of-control warnings. |
| v3.3 | 2026-05-14 | 6-pass production-readiness audit. App renamed from "Paratech Strut Selector" to FieldStruts. Extension rule fixes, error handling improvements, safety checks in selection algorithm. |
| v3.2 | 2026-05-11 | Customizable apparatus types (add, rename, reorder, remove) from Settings. |
| v3.1 | 2026-05-11 | Dynamic ICS org chart: custom sub-roles, rename roles, remove roles, tree-based rendering at any depth. |
| v3.0 | 2026-05-11 | Drag-and-drop ICS org chart repositioning (tap-to-swap, desktop drag, touch drag). |
| v2.4 | 2026-05-11 | Apparatus type hierarchy (Chief through Other). Apparatus groups for combining units during operations. |
| v2.3 | 2026-05-11 | NIMS/ICS terminology update ("Team" to "Group"). Collapsible operation sections. Quick Find defaults to no system preselected. |
| v2.2 | 2026-05-11 | Interactive ICS org chart (tap any node to assign). Grouped shore points advance together. |
| v2.1 | 2026-05-11 | Cut measurement persistence. Actual vs. expected cut display. Label clarity improvements. |
| v2.0 | 2026-05-11 | Production hardening: Firebase error handling with offline write queue, XSS protection, safe localStorage writes. |
| v1.11 | 2026-05-10 | Pending shore points (save when no equipment available, auto-detect availability). ICS/NIMS hierarchy chart. |
| v1.10 | 2026-05-10 | Individual personnel tracking with ICS role assignment. |
| v1.9 | 2026-05-10 | External equipment editing after creation. Quick Add grid for external equipment. Drill-down stops at Area level. |
| v1.8 | 2026-05-10 | 6-status shore point workflow (In Process through Removed & Returned). |
| v1.7 | 2026-05-10 | ICS drill-down navigation (Building > Division > Floor > Team). 3-tier view system (Operations, Command, Cut Table). |
| v1.4 | 2026-05-08 | Live inventory count badges on Quick Add. |
| v1.1 | 2026-05-07 | Department login screen. Multi-strut deploy (1x-4x per shore point). |
| v1.0 | 2026-05-07 | First release. Quick Select, apparatus management, inventory tracking, live operations. |